Industrial Machine Failure Prediction Using Machine Learning

1. Abstract

Industrial machines are continuously exposed to varying operating conditions, and unexpected machine failures can lead to production delays, maintenance costs, and equipment downtime. This project presents a machine learning-based approach for predicting industrial machine failure from operating-condition data. The AI4I 2020 Predictive Maintenance Dataset is used for analysis and model development. The dataset contains machine operating measurements including air temperature, process temperature, rotational speed, torque, tool wear, and machine type.

The data is explored and preprocessed before developing classification models. Logistic Regression, K-Nearest Neighbors (KNN), and Decision Tree models are evaluated using Accuracy, Precision, Recall, F1-score, Confusion Matrix, and ROC-AUC. Since machine-failure observations are much fewer than non-failure observations, class imbalance is also considered using balanced Logistic Regression and Decision Tree models. The final system is demonstrated through a simple Streamlit application, where users can enter machine operating conditions and receive a failure prediction and probability. The project demonstrates how machine learning can support predictive maintenance by identifying conditions associated with potential machine failure.

Keywords: Machine Learning, Predictive Maintenance, Industrial Systems, Machine Failure, Classification, Logistic Regression, KNN, Decision Tree, Streamlit.

2. Introduction

Industrial equipment plays an important role in manufacturing and production environments. Machine failure during operation can interrupt production and may result in maintenance expenses and downtime. Traditional maintenance approaches may depend on fixed maintenance schedules or respond only after a failure occurs.

Predictive maintenance uses machine and sensor-related information to identify patterns associated with equipment failure. Machine learning can be applied to historical operating data to learn these patterns and predict whether a machine is likely to experience failure.

This project focuses on developing an educational machine learning prototype for industrial machine failure prediction. The system uses machine operating parameters as inputs and predicts whether a machine failure is likely to occur.

The project follows an end-to-end workflow involving data collection, exploratory data analysis, preprocessing, model development, evaluation, improvement for class imbalance, and application development.

3. Problem Definition

Industrial machines operate under different temperatures, rotational speeds, torque levels, and tool-wear conditions. These conditions can be related to machine failures.

The problem addressed in this project is:

To develop a machine learning classification system that predicts whether an industrial machine will experience failure based on its operating conditions.

The prediction problem is formulated as a binary classification problem:

- 0 → No Machine Failure

- 1 → Machine Failure

4. Objectives

The main objectives of the project are:

- To analyze an industrial predictive-maintenance dataset.

- To identify relevant machine operating features.

- To perform exploratory data analysis and preprocessing.

- To develop Logistic Regression, KNN, and Decision Tree classification models.

- To evaluate the models using suitable classification metrics.

- To address the class imbalance present in machine-failure data.

- To develop a simple Streamlit prototype for machine-failure prediction.

- To provide a reproducible machine learning workflow that can be explained and demonstrated.

5. Related Work

Predictive maintenance is an important application area of machine learning in industrial systems. Machine learning methods can be used to analyze historical equipment data and identify patterns associated with abnormal operating conditions or failures.

Classification algorithms such as Logistic Regression, KNN, and Decision Trees can be used when the objective is to assign observations to failure or non-failure classes. Different algorithms can produce different results depending on the characteristics of the dataset.

An important challenge in machine-failure prediction is class imbalance, because failures are generally less frequent than normal machine operation. Therefore, accuracy alone may not sufficiently describe model performance. Precision, Recall, F1-score, and ROC-AUC can provide additional information about classification performance.

For this project, foundational machine learning methods were selected to maintain a simple, interpretable, and reproducible educational prototype.

6. Dataset

6.1 Dataset Name

AI4I 2020 Predictive Maintenance Dataset

6.2 Dataset Source

The dataset was obtained from Kaggle and is based on the AI4I 2020 predictive maintenance dataset.

6.3 Dataset Characteristics

The dataset contains:

- 10,000 observations

- 14 columns

- No missing values

The important columns include:

| Feature | Description |
| --- | --- |
| Type | Machine/product type |
| Air temperature [K] | Air temperature |
| Process temperature [K] | Process temperature |
| Rotational speed [rpm] | Machine rotational speed |
| Torque [Nm] | Applied torque |
| Tool wear [min] | Tool usage/wear |
| Machine failure | Target variable |

Other columns include failure-mode indicators such as TWF, HDF, PWF, OSF, and RNF.

6.4 Target Variable

The target variable is Machine failure.

- 0 → No failure

- 1 → Failure

The dataset contains 9,661 non-failure observations and 339 failure observations, showing a strong class imbalance.

7. Data Preprocessing

The following preprocessing steps were performed.

7.1 Feature Selection

The following features were selected:

Type

Air temperature [K]

Process temperature [K]

Rotational speed [rpm]

Torque [Nm]

Tool wear [min]

UDI and Product ID were excluded because they are identifiers rather than meaningful operating features.

7.2 Avoiding Data Leakage

The failure-mode indicator columns:

TWF

HDF

PWF

OSF

RNF

were excluded from the main model features because they directly indicate specific machine-failure modes and could introduce data leakage into the prediction task.

7.3 Encoding

The categorical Type feature was converted into numerical form using one-hot encoding.

X = pd.get_dummies(X, columns=['Type'], drop_first=True)

7.4 Train-Test Split

The dataset was divided into:

- 80% training data

- 20% testing data

Stratified splitting was used so that the class distribution was preserved.

7.5 Feature Scaling

Standardization was applied for models that require scaled numerical features, particularly Logistic Regression and KNN.

from sklearn.preprocessing import StandardScaler

scaler = StandardScaler()

X_train_scaled = scaler.fit_transform(X_train)

X_test_scaled = scaler.transform(X_test)

8. Exploratory Data Analysis

Exploratory Data Analysis was performed to understand the dataset and identify relationships between operating conditions and machine failure.

The analysis included:

- Dataset structure

- Data types

- Missing-value checking

- Target-class distribution

- Feature distributions

- Correlation analysis

- Comparison of operating conditions between failure and non-failure cases

The target distribution showed that machine failures represented only a small portion of the observations.

The mean comparison showed differences in several operating variables.

| Feature | No Failure Mean | Failure Mean |
| --- | --- | --- |
| Air Temperature [K] | 299.974 | 300.886 |
| Process Temperature [K] | 309.996 | 310.290 |
| Rotational Speed [rpm] | 1540.260 | 1496.487 |
| Torque [Nm] | 39.630 | 50.168 |
| Tool Wear [min] | 106.694 | 143.782 |

These differences indicate that variables such as torque and tool wear may provide useful signals for classification. However, these comparisons show association in the dataset and do not establish a causal relationship.

9. Model Development

Three foundational machine learning algorithms were developed.

9.1 Logistic Regression

Logistic Regression was used as a baseline classification model.

from sklearn.linear_model import LogisticRegression

logistic_model = LogisticRegression(

random_state=42,

max_iter=1000)

logistic_model.fit(X_train_scaled, y_train)

y_pred_lr = logistic_model.predict(X_test_scaled)

The model predicts the probability of an observation belonging to the machine-failure class.

9.2 K-Nearest Neighbors

KNN classifies an observation based on the classes of nearby observations.

A value of k = 5 was used.

from sklearn.neighbors import KNeighborsClassifier

knn_model = KNeighborsClassifier(n_neighbors=5)

knn_model.fit(X_train_scaled, y_train)

y_pred_knn = knn_model.predict(X_test_scaled)

9.3 Decision Tree

A Decision Tree was developed to learn decision rules from the machine operating features.

from sklearn.tree import DecisionTreeClassifier

decision_tree_model = DecisionTreeClassifier(

random_state=42,

max_depth=5

)

decision_tree_model.fit(X_train, y_train)

y_pred_dt = decision_tree_model.predict(X_test)

The maximum depth was limited to 5 to control model complexity.

10. Class Imbalance Handling

The dataset contains considerably more non-failure observations than failure observations.

To investigate the effect of class imbalance, balanced versions of Logistic Regression and Decision Tree were trained using class_weight='balanced'.

Balanced Logistic Regression

logistic_balanced = LogisticRegression(

class_weight='balanced',

random_state=42,

max_iter=1000

)

logistic_balanced.fit(X_train_scaled, y_train)

Balanced Decision Tree

decision_tree_balanced = DecisionTreeClassifier(

class_weight='balanced',

max_depth=5,

random_state=42

)

decision_tree_balanced.fit(X_train, y_train)

The balanced models give greater importance to the minority failure class during training.

11. Model Evaluation

The classification models were evaluated using:

Accuracy

Measures the overall proportion of correct predictions.

Precision

Measures how many observations predicted as failures were actually failures.

Recall

Measures how many actual failures were correctly detected.

F1-Score

Provides a combined measure of Precision and Recall.

Confusion Matrix

Shows:

- True Positives

- True Negatives

- False Positives

- False Negatives

ROC-AUC

Measures the model's ability to distinguish between failure and non-failure classes across different classification thresholds.

12. Model Comparison

The models evaluated in this project are:

- Logistic Regression

- Balanced Logistic Regression

- KNN

- Decision Tree

- Balanced Decision Tree

The final results should be reported using the values obtained from the Colab implementation.

| Model | Accuracy | Precision | Recall | F1-Score | ROC-AUC |
| --- | --- | --- | --- | --- | --- |
| Logistic Regression | 0.9675 | 0.6364 | 0.1029 | 0.1772 | 0.8995 |
| Balanced Logistic Regression | 0.8245 | 0.1418 | 0.8235 | 0.2419 | 0.9070 |
| KNN | 0.9740 | 0.8333 | 0.2941 | 0.4348 | 0.8292 |
| Decision Tree | 0.9730 | 0.7692 | 0.2941 | 0.4255 | 0.9147 |
| Balanced Decision Tree | 0.9295 | 0.3109 | 0.8824 | 0.4598 | 0.9079 |

Important: Do not fill these values with guessed numbers. Use the actual results from your Colab notebook.

13. Discussion

The experimental results demonstrate that different classification algorithms behave differently when predicting machine failure.

Because machine failures are relatively rare, accuracy should not be considered independently. A model that predicts most observations as non-failure can achieve high accuracy while failing to detect a substantial number of actual failures.

Therefore, Recall and F1-score for the failure class, together with ROC-AUC and the confusion matrix, are important when interpreting the results.

The balanced models were investigated to determine whether giving additional importance to the minority failure class improves the detection of failures.

The final model should be selected based on the actual evaluation results and the project's objective of identifying machine failures.

14. Error Analysis

Error analysis was performed using the confusion matrices.

The main types of prediction errors are:

- False Positive: The model predicts failure when the machine does not fail.

- False Negative: The model predicts no failure when the machine actually fails.

For predictive maintenance, false negatives are particularly important to investigate because they represent actual failures that were not detected by the model.

The confusion matrix of the selected final model should therefore be included in the final paper.

15. Prototype Development

A lightweight Streamlit application was developed to demonstrate the trained machine learning model.

The application accepts:

- Machine Type

- Air Temperature

- Process Temperature

- Rotational Speed

- Torque

- Tool Wear

The entered values undergo the same preprocessing used during model training. The trained model then generates:

- Machine failure prediction

- Failure probability

The application provides a simple interface for demonstrating the model with new machine operating conditions.

The capstone specification requires a Streamlit prototype that validates inputs, applies the training preprocessing, and produces a clear prediction.

16. System Workflow

The complete system follows this workflow:

Machine Operating Data

↓

Data Cleaning

↓

Feature Selection

↓

One-Hot Encoding

↓

Train-Test Split

↓

Feature Scaling

↓

Model Development

↓

Model Evaluation

↓

Class Imbalance Handling

↓

Final Model

↓

Streamlit Application

↓

Machine Failure Prediction

17. Implementation Tools and Technologies

| Category | Technology |
| --- | --- |
| Programming Language | Python |
| Development Environment | Google Colab |
| Data Processing | Pandas, NumPy |
| Visualization | Matplotlib, Seaborn |
| Machine Learning | Scikit-learn |
| Model Serialization | Joblib |
| Web Application | Streamlit |
| Dataset | AI4I 2020 Predictive Maintenance Dataset |

18. Limitations

The project has the following limitations:

- The model is developed using a single predictive-maintenance dataset.

- The dataset may not represent every type of industrial machine or manufacturing environment.

- The system is an educational prototype rather than a production deployment.

- The prediction depends on the quality and range of the input operating conditions.

- The model does not replace physical inspection or professional maintenance procedures.

19. Future Scope

The project can be extended in several ways:

- Integrate real-time sensor data from industrial machines.

- Include additional sensor parameters such as vibration and pressure where appropriate.

- Evaluate additional machine learning algorithms.

- Perform more extensive hyperparameter tuning.

- Develop continuous monitoring and alert mechanisms.

- Deploy the application for controlled real-time predictive-maintenance demonstrations.

- Retrain the model periodically using newly collected machine data.

20. Conclusion

This project developed an end-to-end machine learning approach for industrial machine failure prediction. The AI4I 2020 Predictive Maintenance Dataset was analyzed and relevant machine operating features were selected for classification.

Logistic Regression, KNN, and Decision Tree models were developed and evaluated using Accuracy, Precision, Recall, F1-score, Confusion Matrix, and ROC-AUC. Class imbalance was also addressed using balanced Logistic Regression and Decision Tree models.

The final system was integrated into a Streamlit application that accepts machine operating conditions and provides a machine-failure prediction with failure probability. The project demonstrates the complete workflow from data analysis and model development to evaluation and application-based prediction.

21. References

Use the actual sources you consulted. At minimum, include the dataset source and the relevant machine-learning documentation.

Dataset

- AI4I 2020 Predictive Maintenance Dataset, Kaggle.
Dataset used for machine operating-condition and failure prediction.

Documentation

- Scikit-learn documentation — Machine Learning in Python.

- Pandas documentation — Data analysis and manipulation using Python.

- Streamlit documentation — Python framework for building data applications.

Project Specification

- Learn Depth Academy LLP, Track 1 Final Capstone — Industrial Machine Failure Prediction, provided capstone specification.